Abstract:

Rates of Gulf of Mexico shoreline change are calculated on the basis of a linear

regression of past shoreline positions. A computer program called the Shoreline

Shape and Projection Program (SSAPP), developed by the Bureau of Economic

Geology of The University of Texas at Austin, was used to calculate the rate of

shoreline change every 82 ft (25 m) alongshore. SSAPP automatically draws a

segmented baseline that follows the mean position of historical shorelines.

Transects that intersect the shorelines are constructed perpendicular to this

baseline. Distances between the shoreline positions along each transect are

determined, and a linear regression model is used to calculate the average

annual rate of shoreline change.

The following historical shorelines were used in the analysis: 1941, 1956, 1982, 1995

Purpose:

State and Federal agencies with coastal management responsibilities

currently rely on average rates of shoreline movement and projected future

shoreline positions for regulatory purposes.  As a result of this dependency

on scientific data, regional studies of shoreline movement are now regarded

as important sources of information for formulating coastal mangement

policies and long range planning.  These coastal investigations now serve

as a primary technical basis for decisions made by coastal planners and

managers of natural resources located near the shore.

Limitations_of_Data:

Entity_and_Attribute_Overview:

-TRAN

Transect number output from SSAPP.  For each shoreline segment, numbers start

with the most negative number on the north end and increase to the south.

-LR_M_YR_

Calculated erosion rate using the linear regression method, in meters/yr.

-LR_FT_YR_

Calculated erosion rate using the linear regression method, in feet/yr.

-EASTING_UTM

Longitudinal coordinate of shoreline change point in UTM, from GPS.

-NORTHING_UTM

Latitudinal coordinate of shoreline change point in UTM, from GPS.

-S1 through S10

Actual years of shorelines used in the rate of change calculation.

Procedures_Used for shoreline mapping:

Mapped shorelines spanning from 1941 to 1982 were created by digitizing a line interpreted from a series of georectified historic aerial photographs.  The 1995 shoreline was directly interpreted and digitized on color infra red digital orthophoto quarter quadrangles.

The Gulf erosion points were imported into ArcView as a text file

conversion. The original points and attribute information being

SSAPP output. Projection and datum information was checked to

affirm correct placement of points along the shore.  In-house quality

checking was completed.

Revisions:

This dataset is version 1.0 and has no predecessor.

Reviews_Applied_to_Data:

In-house review covered in 'Purpose' above

Related_Spatial_and_Tabular_Data_Sets:

None

References_Cited:

Notes:

Currentness_Reference:

This reference document was updated on 3-6-2003.

Maintenance_and_Update_Frequency:

None Planned

Access_Constraints:

None

Data_Set_Credit:

Rachel Waldinger

Lying Xu

James Gibeaut

Tom Tremblay

Completeness_Report:

Complete

Horizontal_Positional_Accuracy_Report:

90 percent of well defined points fall within 40 feet of their true position

Vertical_Positional_Accuracy_Report:

N/A

Cloud_Cover:

N/A