Abstract:

Storm washover features of the Texas coast. Features include

washover channels, interdune drainages, and washover

areas. Channels and drainages are represented as polygon

features while washover areas are represented as linear

shoreline features.

Purpose:

The dataset was created as part of the Texas Natural

Resource Inventory(NRI) and is intended for oil spill

response and identification of coastal hazards.

Limitations_of_Data:

The data were captured at 1:24,000 scale. Original

washover mapping was modified by the Texas General

Land Office to conform with that agencies standard

Texas shoreline dataset. The relative position between

shoreline and washover features may change due to

coastal modification.

Entity_and_Attribute_Overview:

Washover-id in the .PAT is coded as to the type of

washover feature. Washover channels are coded 2 and

interdune drainages are coded 3. Washover.AAT contains

the item ID (4 5 B). Shorelines which delimit washover

areas are coded as ID 99. All other arcs are coded 9999.

Procedures_Used:

Washover features were interpreted from 1992 aerial

videography supplemented with early 1990s aerial

photography and zoom transferred to USGS 7.5'

quadrangles by Bill White. The quads were then digitized,

coded as to washover feature type, and transformed and

projected by Tom Tremblay.

Revisions:

1. Original coverage provided to the Texas General

Land Office by the Bureau of Economic Geology.

2. Washover features adapted to GLO shoreline.

3. GLO washover coverage acquired by the BEG and edited.

Reviews_Applied_to_Data:

Check plots of the original data, with feature labels,

were plotted to scale and quality checked by Bill White.

The GLO version of the dataset was acquired and checked

against the original by Tom Tremblay. Data were compared

to DOQ data by James Gibeaut.